无刷直流行星齿轮减速电机(内齿圈材质:金属)

42JX150K/42ZWNP66

Φ42行星减速器,允许转矩范围:2N.m~15N.m

42mm OD Planetary Gearbox,

Permissible Load Range: 2N.m~15N.m

BLDC Planetary Gear Motor (Ring Gear Material: Metal)

无刷直流电动机，3相10极，内置驱动器 3 phase 10 poles brushless DC motor, with built-in driver

- 外观和安装尺寸 Dimensions

  - 无刷电动机主要技术参数Brushless Motor Technical Data

| 电机型号 TYPE | 极数 Number of poles | 相数 Number of phase | 额定电压 Rated voltage VDC | 空载转速 No-load speed r/min | 额定转速 Rated speed r/min | 额定转矩 Rated torque mN.m | 额定电流 Rated current A | 输出功率 Output power W |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42ZWNP66-2430 | 10 | 3 | 24 | 3000 | 2550 | 80 | 1.1 | 21 |

  - 无刷行星齿轮减速电机主要技术参数Brushless Gear Motor Technical Data

配置42ZWNP66-2430无刷直流电机(42ZWNP66-2430 BLDC Motor)

| 减速比 Reduction ratio | 3.5 | 14 | 21 | 49 | 84 | 104 | 126 | 156 | 172 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 减速级数 Number of gear trains | 1 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 |
| 减速器长度(L) Length(L) mm | 32.5 | 39.2 | 39.2 | 46 | 46 | 46 | 46 | 52.5 | 52.5 |
| 空载速度 No-load speed r/min | 857 | 214 | 143 | 61 | 36 | 29 | 24 | 19 | 17 |
| 额定转速 Rated speedrAnin | 728 | 182 | 121 | 52 | 30 | 24 | 20 | 16 | 15 |
| 额定转矩 Rated torqueN.m | 0.25 | 0.9 | 1.3 | 2.8 | 4.9 | 6.0 | 7.3 | 8.2 | 9.0 |
| 最大瞬间允许负载N.m Max. permissible load in a short time | 3 | 12 | 12 | 25 | 25 | 25 | 25 | 30 | 30 |

- 接线方式Wire Diagram

| 正极+ Phase V | 负极- Phase W | 转向 Reversing | 调速 Speed Control | 制动 Braking | +5V输出 Output | 脉冲输出 Pulse Output |
| --- | --- | --- | --- | --- | --- | --- |
| 红 | 黑 | 棕 | 橙 | 白 | 绿 | 蓝 |
| Red | Black | Brown | Orange | White | Green | Blue |

棕色与黑色导通为CCW,断开为CW。

白色与黑色导通为停止。

橙色为调速功能。

绿色与棕色导通全速工作。

蓝色肩舌脉冲输出，电机每转一周为5个脉冲输出。

Brown wire connects with black wire for CCW and disconnects for CW.

White wire connects with black wire for stopping.

Orange wire is for speed control.

Green wire connects with brown wire for full-speed working.

Blue wire is for pulse output, motor outputs 5 pulses per revolution.

(Green ) PWM 绿色(Green)

- FG 绿色(Green )